Washing Machine Project Documentation

1. CHAPTER 1 : PROJECT SPECIFICATIONS

Implement a small controller for a washing machine implemented on FPGA, having the capability of executing manual and automatic wash with configurable temperature, spin speed, prewash, and rinse as options.

Operating Procedure:

The washing machine controller is designed to move a laundry cycle from user input to final spin and unlocking. The system uses a Moore finite state machine (FSM) to control steps such as filling water, heating, washing, rinsing, and spinning.

The machine can operate in two modes: automatic and manual. In automatic, temperature, time, spin speed, and rinse are preselected by programs. In manual mode, parameters are selected directly by the user.

The following steps describe a typical automatic cycle:

________________________________________

Step 1 – Initialization (IDLE state)

•\\tOn power-up, the machine goes to the IDLE state.

•\\tAll outputs are not active.

•\\tThe system waits for door closure and press of the START button.

________________________________________

Step 2 – Selection of Automatic Mode

• One of the preprogrammed automatic modes is selected by the user with the mode selector:

• Quick Wash, Shirts, Dark Colors, Dirty Laundry, Antiallergic

• The system derives temperature, spin speed, prewash, and extra rinse settings from this selection.

________________________________________

Step 3 – Locking of Door and Prewash Decision

• When START is turned on and the door is closed:

• The door is locked.

• If prewash is required under the selected mode or manual program, the system proceeds to PREWASH.

o

Otherwise, it proceeds to FILL_WATER.

________________________________________

Step 4 – Water Filling (FILL_WATER state)

•

The washing machine fills the drum with water.

•

A sensor verifies that water level is sufficient (water_level_ok = '1').

________________________________________

Step 5 – Heating Water (HEAT_WATER state)

•

The system activates the heater.

•

The water temperature increases in 1°C steps every 2 seconds to the set temperature.

________________________________________

Step 6 – Washing (WASH state)

•

The motor starts washing the drum.

• A timer initiates countdown based on the mode (e.g., 10–30 minutes).

• The system decides after completion if a rinse is necessary.

______________________________________

Step 7 – Rinsing (RINSE state, if enabled)

• Water is replenished in the drum.

• A rinse timer begins.

• The motor runs again to wash clothes.

______________________________________

Step 8 – Draining (DRAIN state)

• Water is drained using a pump after washing/rinsing.

• The drum is drained before it spins.

______________________________________

Step 9 – Spinning (SPIN state)

• The motor enters the high-speed spinning mode.

•	Depends on the selected mode and rotation speed (800–1200 RPM).

________________________________________

Step 10 – Finish and Unlock (FINISH, UNLOCK states)

•	All machine operations are halted.

•	Door is unlocked.

•	System returns to IDLE and is ready for a new cycle.

2.  CHAPTER 2: DESIGN

2.1. Black Box

Inputs:

- start_button, door_closed

- mode_select, temp_select, prewash_select

- water_level_ok

Outputs:

- HEATER, MOTOR, DRAIN_MODE, LED_MODE, DISPLAY

- door_locked

The black box defines the external behavior of the washing machine system by all inputs and outputs, regardless of how they are implemented internally. This exercise is about imagining how the user and environment interact with the system and what hardware constraints there are on the FPGA board.
The aim is to get all use cases (potential interactions or events), and from them, determine all input and output signals, both those mentioned explicitly and those implied but necessary for proper functioning.
Inputs such as START_BUTTON, DOOR_CLOSED, PREWASH_SELECT, and WATER_LEVEL_OK are not necessarily explicitly provided in problem statements, but must exist to simulate real hardware. LED_MODE and DISPLAY provide feedback to the user, which is critical in embedded systems and typically overlooked except where visualized from real user input. CLK and RESET are fundamental for timing operation and FSM reset. DOOR_LOCKED and MOTOR are safety outputs, with the machine being safe when operating.

2.2. Control and Execution Unit

The system black box must be decomposed further to find pieces that can be implemented. We will decompose top-down to reach known circuits, and we will implement bottom-up.
The first decomposition of the washing machine controller decomposes the system into two large functional blocks:

- Control Unit (CU) – implements the control logic via a finite state machine (FSM).

- Execution Unit (EU) – contains the physical resources (timers, heaters, motors, etc.) and executes commands issued by the CU.

This modular decomposition follows classic digital design principles and simplifies implementation and testing.

Control Unit (CU)

The Control Unit is responsible for:

- Monitoring the system’s current state

- Reacting to user inputs (START_BUTTON, MODE_SELECT, DOOR_CLOSED)

- Generating control signals to coordinate each phase of the wash cycle

- Advancing through the FSM based on timer expirations, sensor inputs (WATER_LEVEL_OK, temperature), and wash completion

The CU is designed as a Moore finite state machine where:

- Each state represents a phase (e.g., FILL_WATER, WASH, SPIN)

- Transitions depend on inputs and internal flags (e.g., prewash enabled)

- Outputs are tied to the current state, providing predictable behavior

Execution Unit (EU)

The Execution Unit performs all data-path and physical operations required to execute the wash cycle:

It includes:

- Temperature Controller: A simulated counter that increases current_temperature toward a target_temperature, using a 2-second step

- Motor Controller: Drives the drum in wash or spin mode, depending on motor_control

- Water Level Sensor Input: Simulates tank filling or draining (WATER_LEVEL_OK)

- Heater Control: Activates heating when required

- Timers: Countdown for wash time, spin duration, prewash, and rinse

- Display Encoder: Converts current countdown timer into a 7-segment display value

- Door Lock: Ensures door cannot be opened during operation

CU-EU Interaction

The CU sends enable signals and state commands to the EU:

| Control Signal |  |  |  |  |  |  |  |  | Sent From CU |  |  |  |  |  |  |  |  |  |  | Action Performed by EU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HEATER |  |  |  |  |  |  |  |  | State = HEAT |  |  |  |  |  |  |  |  |  |  | Increments temperature |
| MOTOR |  |  |  |  |  |  |  |  | State = WASH/SPIN |  |  |  |  |  |  |  |  |  |  | Drives motor in selected mode |
| DRAIN_MODE |  |  |  |  |  |  |  |  | State = DRAIN |  |  |  |  |  |  |  |  |  |  | Starts water pump |
| DISPLAY_EN |  |  |  |  |  |  |  |  | State Active |  |  |  |  |  |  |  |  |  |  | Updates timer shown |
| DOOR_LOCKED |  |  |  |  |  |  |  |  | IDLE vs RUN |  |  |  |  |  |  |  |  |  |  | Locks/unlocks door |

The EU reports back status signals used by the CU to make transitions:

| Status Signal | From EU to CU | Meaning |
| --- | --- | --- |
| WATER_LEVEL_OK | From sensor | Water filled or empty |
| current_temp | From temp counter | Reached desired temperature |
| TIMER = 0 | From any timer | Phase time completed |

2.2.1. Mapping the inputs and outputs of the big box on the two components UC and UE.

CU–EU separation makes the washing machine design:

- Modular: You can change the EU implementation without modifying the CU

- Readable: The FSM remains clear, without being cluttered by timer or motor logic

- Expandable: You can later add features like a buzzer, display, or IoT without impacting state logic

To implement the washing machine controller correctly, we divide the inputs and outputs into two distinct categories:

1. Inputs

🟦 Data Inputs

These are values selected by the user or system that define how the washing cycle should behave:

| Signal Name |  |  |  |  |  |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEMP_SELECT[1:0] |  |  |  |  |  |  |  |  |  | Temperature selection (manual mode: 30, 40, 60°C) |
| MODE_SELECT[2:0] |  |  |  |  |  |  |  |  |  | Automatic program selection (quick, shirts, etc.) |
| SPEED_SELECT[1:0] |  |  |  |  |  |  |  |  |  | Spin speed selection (800, 1000, 1200 rpm) |

🟧 Control Inputs

These are signals that tell the system what to do or when to begin/stop an action:

| Signal Name | Description |
| --- | --- |
| START_BUTTON | Confirms the selected program and starts cycle |
| PREWASH_SELECT | Requests an additional prewash cycle |
| RINSE_SELECT | Requests an extra rinse |
| DOOR_CLOSED | Indicates the door is physically shut |
| RESET | Resets the system to the initial IDLE state |
| WATER_LEVEL_OK | Comes from a water-level sensor (fill/empty) |
| CLK | System clock for timers and FSM transitions |

2. Outputs

🟦 Data Outputs

These are values or information presented to the user or used internally to reflect system data:

| Signal Name | Description |
| --- | --- |
| DISPLAY[6:0] | 7-segment display showing remaining time |
| target_temperature | Current target temperature for water |
| spin_speed | Currently selected spin speed |

🟧 Control Outputs

These outputs control devices or signal status to the user during operation:

| Signal Name | Description |
| --- | --- |
| HEATER | Activates heating element |
| MOTOR | Drives the drum for washing or spinning |
| DRAIN_MODE | Controls the drain pump |
| LED_MODE[1:0] | Indicates the current cycle phase (wash/rinse/etc.) |
| DOOR_LOCKED | Locks the door during operation for safety |

2.2.2. Establishing the Resources (EU)

To establish a working link between the Execution Unit (EU) and the Control Unit (CU), we must first identify the resources utilized to perform washing machine operations. These resources either provide information to the CU (status or condition signals) or obtain commands from the CU (enable signals, resets, data inputs).

All the decisions in the FSM are being taken on the basis of status information from the EU, and therefore the EU must possess resources that can generate this decision-making information.

--pt ajutor la resurcess

| Resource | Type | CU Control | CU Input from EU |
| --- | --- | --- | --- |
| Temperature Counter | Simple | Enable/Reset Heater | current_temperature |
| General Timer | Simple (counter) | Reset, Decrement | TIMER = 0 |
| Wash Timer | Simple (counter) | Reset, Decrement | wash_timer = 0 |
| Prewash Timer | Simple (counter) | Reset, Decrement | prewash_timer = 0 |
| Rinse Timer | Simple (counter) | Reset, Decrement | rinse_timer = 0 |
| Spin Timer | Simple (counter) | Reset, Decrement | spin_timer = 0 |
| Water Level Sensor | Physical (external) | — | water_level_ok |
| Heater | Actuator | ON/OFF | — |
| Motor Controller | Actuator | Mode Control (wash/spin) | — |
| Drain Pump | Actuator | ON/OFF | — |
| Door Lock | Actuator | ON/OFF | — |
| 7-Segment Display Encoder | Combinational logic | Data (time value) | — |

Information Flow: CU–EU Interaction

⬅️ From EU to CU (Decision Inputs)

These resources generate status conditions that drive transitions in the FSM:

- current_temperature: from the temperature counter; used to decide when to move from HEAT_WATER to WASH

- TIMER = 0: used in all timed phases (WASH, RINSE, SPIN) to signal completion

- WATER_LEVEL_OK: sensor input to determine transition from FILL_WATER to HEAT_WATER or DRAIN to SPIN

➡️ From CU to EU (Control Signals)

The CU sends enable or mode signals to resources to activate their functionality:

- Enable Heater: only during HEAT_WATER, until temperature reached

- Start Motor: during WASH, RINSE, or SPIN

- Enable Drain: during DRAIN state

- Lock Door: from START until UNLOCK state

- Display Timer: selected timer value based on active state

🧩 Example: Heating System

Abstract block:

- Inputs: Enable, Target_Temperature, Current_Temperature

- Output: Temperature_Reached flag

Decomposition:

- Counter that increments every 2 seconds if enabled

- Compares current value to target value

- Generates flag to CU when match is reached

🔍 Why Resource Identification Matters

This analysis ensures that every transition condition in the FSM is backed by an actual implemented resource. It also:

- Provides a clean interface between CU and EU

- Simplifies testing and modular design

- Allows each resource (e.g., timers) to be reused in multiple states

2.2.3. Block Diagram for first breakdown

2.3. State diagram of Control Unit

--o avem

2.4. Detailed diagram

CHAPTER 3: USER MANUAL

This user manual describes the step-by-step operation of the FPGA-controlled washing machine. It explains the roles of physical inputs, expected feedback from outputs (LEDs, display), and system behavior at each stage. It is designed for a user interacting with physical switches and indicators on an FPGA board, simulating the interface of a real washing machine.

3.1 System Startup & Reset

- After powering the board, the system initializes and enters the IDLE state.

- All actuators (HEATER, MOTOR, DRAIN) are turned off.

- The 7-segment display shows 0 or is blank.

- The LED_MODE output is set to "00" to indicate an idle/ready state.

- If the board is in an unknown state or has been interrupted, press the RESET button to reinitialize the controller.

🧺 3.2 Preparing the Machine

- Open the machine door (simulated by a toggle switch connected to DOOR_CLOSED).

- Load clothes into the drum (physical step).

- Close the door – ensure the DOOR_CLOSED signal is HIGH ('1').

- Select operation mode:

  - If manual mode (MODE_SELECT = "000"):

    - Choose temperature with TEMP_SELECT[1:0]

      - 00 → 30°C

      - 01 → 40°C

      - 10 → 60°C

      - 11 → 90°C

    - Optionally activate:

      - PREWASH_SELECT (1 = prewash enabled)

      - RINSE_SELECT (1 = extra rinse enabled)

      - (Optional: spin speed via SPEED_SELECT)

  - If automatic mode:

    - Select program using MODE_SELECT[2:0]:

| Code | Program | Temp | Speed | Prewash | Extra Rinse |
|------|----------------|------|--------|---------|-------------|
| 001 | Quick Wash | 30°C | 1200 | No | No |
| 010 | Shirts | 60°C | 800 | No | No |
| 011 | Dark Colors | 40°C | 1000 | No | Yes |
| 100 | Dirty Laundry | 40°C | 1000 | Yes | No |
| 101 | Antiallergic | 90°C | 1200 | No | Yes |

3.3 Starting the Wash Cycle

- Press the START_BUTTON to initiate the wash cycle.

- The controller verifies:

  - Door is closed

  - A valid mode is selected

- The door is locked automatically (DOOR_LOCKED output goes HIGH).

- The system transitions to the first required state:

  - PREWASH (if enabled), otherwise

  - FILL_WATER

3.4 Wash Cycle Phases

The FSM progresses through the following phases automatically. Each state has distinct outputs and timing:

🔄 FILL_WATER State

- Activates water intake until WATER_LEVEL_OK = '1'.

- Display remains idle or shows a loading symbol.

- LED_MODE = "01" (indicating active).

♨️ HEAT_WATER State

- Heater is activated (HEATER = '1').

- Temperature increases 1°C every 2 seconds.

- When current_temperature = target_temperature, the state transitions to WASH.

🧼 PREWASH (if selected)

- Motor rotates drum slowly (MOTOR = '1').

- Countdown begins for prewash duration (e.g., 5 minutes).

- After time expires, machine transitions to DRAIN.

🧽 WASH State

- Main wash phase: motor rotates the drum.

- Timer countdown based on mode selection (10–30 minutes).

- DISPLAY shows remaining seconds.

- After wash ends:

  - If extra_rinse_needed = '1', transitions to RINSE

  - Else transitions to DRAIN

💧 RINSE (if selected)

- Water is refilled.

- Motor runs short rinse cycle.

- Timer shown on display.

- Transitions to DRAIN when complete.

🚰 DRAIN State

- DRAIN_MODE = '1' activated to empty water.

- Machine waits until WATER_LEVEL_OK = '0'.

🌀 SPIN State

- High-speed spin based on SPEED_SELECT or mode default

  - 800 RPM → 3 min

  - 1000 RPM → 4 min

  - 1200 RPM → 5 min

- Countdown shown on display

- After spin, machine moves to FINISH

✅ FINISH State

- All components stop.

- Door remains locked momentarily.

- LED_MODE = "00" (done)

🔓 UNLOCK State

- DOOR_LOCKED = '0' – door can now be opened.

- System remains idle until RESET or new START command.

🔢 3.5 Display and Indicators

| Output Signal | Description |
| --- | --- |
| DISPLAY[6:0] | Shows remaining time in seconds |
| LED_MODE[1:0] | Status LED indicating phase: |
| - "00" | Idle / Finished |
| - "01" | Active (washing, rinsing, heating) |
| - "10" | Draining |
| - "11" | Spinning |
| DOOR_LOCKED | High when door is locked |

⚠️ 3.6 Safety and Aborts

- Machine cannot start if the door is open.

- Pressing RESET at any point returns system to IDLE.

- Door remains locked during all active phases.

- In case of error (e.g., WATER_LEVEL_OK fails), machine remains in that state until resolved.

4. CHAPTER 4 : DESIGN SOLUTIONS JUSTIFICATION

--poate ajuta; explica ce face fiecare fisier

Detailed Explanation of the Washing Machine Controller VHDL Implementation

I'll provide a comprehensive breakdown of the complete washing machine controller design, explaining each module's purpose and functionality, along with a detailed description of all variables and signals.

Overall Architecture

The washing machine controller is implemented using a modular design approach with 6 VHDL files:

- washing_machine_pkg.vhd: Package containing shared types, constants, and functions

- spin_decoder_module.vhd: Handles spin cycle speed control via decoding

- display_controller_module.vhd: Manages the 7-segment display and state LEDs

- state_transition_module.vhd: Controls state transitions based on inputs

- signal_handler_module.vhd: Manages washing operation signals and timing

- main.vhd: Top-level entity connecting all components

1. Package File (washing_machine_pkg.vhd)

Purpose

This package defines the shared types, constants, and functions used throughout the system.

Key Components

Types:

- state_type: An enumeration defining the main states of the machine:

  - IDLE: Initial state, waiting for door to close

  - MODE_SELECT: State where mode selection begins

  - CHECK_MODE: Decision point between automatic and manual modes

  - AUTO_PRESET: For selecting one of the automatic programs

  - MANUAL_TEMP: For manual temperature selection

  - MANUAL_SPEED: For manual spin speed selection

  - MANUAL_OPTIONS: For selecting prewash and extra rinse options

  - READY: Configuration complete, ready to start

  - WASHING: Machine is in washing process

- washing_substate_type: Defines sub-states during the washing process:

  - FILL_WATER: Initial water filling

  - PRE_WASH: Optional prewash cycle (same as main wash, 10 min)

  - DRAIN_PRE: Draining after prewash

  - HEAT_WATER: Heating water to set temperature

  - MAIN_WASH: Main washing cycle (20 min at 60 rpm)

  - DRAIN_MAIN: Draining after main wash

  - RINSE_FILL: Filling water for rinse

  - RINSE: Rinse cycle (10 min at 120 rpm)

  - EXTRA_RINSE_CHECK: Check if extra rinse is enabled

  - EXTRA_RINSE: Optional extra rinse cycle

  - DRAIN_RINSE: Draining after rinse(s)

  - SPIN: Final spin cycle

  - COMPLETE: Washing program complete

Constants:

- Temperature constants: TEMP_30C, TEMP_40C, TEMP_60C, TEMP_90C

- Speed constants: SPEED_800, SPEED_1000, SPEED_1200

- Time constants: CLOCK_FREQ, ONE_SECOND, ONE_MINUTE

Functions:

- calculate_time: Calculates total program time based on:

  - Selected temperature (heating time increases with temperature)

  - Prewash option (adds additional heating and wash time)

  - Extra rinse option (adds additional rinse time)

2. Spin Decoder Module (spin_decoder_module.vhd)

Purpose

Translates the spin phase (encoded in 2 bits) into control signals for the motor and drain valve during the spin cycle.

Signals

- Inputs:

  - spin_phase: 2-bit vector representing the current spin phase

    - "00": Start phase

    - "01": Main spin (full speed)

    - "10": Medium speed (slowing down)

    - "11": Final phase (very slow)

- Outputs:

  - spin_outputs: 4-bit vector encoding control signals

    - Bit 0: Drain valve control

    - Bit 1: Motor on/off

    - Bits 2-3: Additional control bits for future expansion

Functionality

- Decodes the spin_phase into appropriate motor and drain valve control signals

- Different phases have different combinations:

  - Start (00): Drain on, motor off - preparing for spin

  - Full spin (01): Drain on, motor on at full speed

  - Medium speed (10): Drain on, motor at medium speed

  - End phase (11): Drain off, motor at low speed before stopping

3. Display Controller Module (display_controller_module.vhd)

Purpose

Manages the 7-segment display and state LEDs based on the current state, showing relevant information to the user.

Signals

- Inputs:

  - current_state: Current main state of the machine

  - washing_substate: Current sub-state during washing

  - selected_temp: Currently selected temperature setting

  - selected_speed: Currently selected speed setting

  - prewash_enabled: Prewash option status

  - extra_rinse_enabled: Extra rinse option status

  - mode_auto: Selected mode (auto or manual)

  - preset_sel: Selected preset in automatic mode

  - remaining_time: Remaining program time

- Outputs:

  - ssd_display: 8-bit output for the 7-segment display

  - state_leds: 4-bit output for state indicator LEDs

Functions

- int_to_ssd: Converts an integer to 7-segment display code

Functionality

- In each state, displays appropriate information:

  - IDLE: Shows "0"

  - MODE_SELECT: Shows "1"

  - CHECK_MODE: Shows "A" for auto or "M" for manual

  - AUTO_PRESET: Shows preset number (1-5)

  - MANUAL_TEMP: Shows temperature (30, 40, 60, 90)

  - MANUAL_SPEED: Shows speed (800, 1000, 1200)

  - MANUAL_OPTIONS: Shows selected options

  - READY: Shows calculated program duration

  - WASHING: Shows remaining time or current washing phase

- Controls state LEDs to indicate current machine state

  - Each state has a unique 4-bit code for LED indication

4. State Transition Module (state_transition_module.vhd)

Purpose

Manages transitions between the main states based on inputs and conditions.

Signals

- Inputs:

  - clk: System clock

  - reset: System reset

  - start_btn: Start button input

  - door_closed: Door closed sensor input

  - mode_auto: Mode selection (auto/manual)

  - config_saved: Configuration saved signal

  - temp_saved, speed_saved, options_saved: Configuration step completion flags

  - washing_substate: Current sub-state during washing

- Outputs:

  - current_state: Current state of the machine

  - next_state: Next state to transition to

Internal Signals

- state_reg: Register holding the current state

- state_next: Signal holding the next state

Functionality

- Implements the state machine transitions based on the organigram

- Key transitions:

  - IDLE → MODE_SELECT: When door is closed

  - CHECK_MODE → AUTO_PRESET or MANUAL_TEMP: Based on mode selection

  - MANUAL_TEMP → MANUAL_SPEED: When temperature is configured

  - MANUAL_SPEED → MANUAL_OPTIONS: When speed is configured

  - MANUAL_OPTIONS → READY: When options are configured

  - AUTO_PRESET → READY: When preset is selected

  - READY → WASHING: When start button is pressed with door closed

  - WASHING → IDLE: When washing is complete

5. Signal Handler Module (signal_handler_module.vhd)

Purpose

Handles all control signals and implements the washing machine operation logic, including timing and phase control.

Signals

- Inputs:

  - clk, reset: Clock and reset signals

  - current_state: Current machine state

  - preset_sel: Automatic preset selection

  - temp_sel, speed_sel: Manual temperature and speed selections

  - prewash_btn, extra_rinse_btn: Option buttons

  - start_btn, door_closed: Control inputs

  - config_saved: Configuration saved signal

- Outputs:

  - washing_substate: Current washing sub-state

  - manual_temp_reg, manual_speed_reg: Manual mode settings

  - auto_temp, auto_speed: Automatic mode settings

  - prewash_enabled, extra_rinse_enabled: Option statuses

  - temp_saved, speed_saved, options_saved: Configuration flags

  - selected_time, remaining_time, counter: Timing values

  - motor_on, water_valve, drain_valve, heater, door_lock: Machine control outputs

  - spin_phase: Current spin phase for decoder

Internal Signals

- wash_substate_reg: Register for washing sub-state

- manual_temp, manual_speed: Storage for manual settings

- auto_temp_reg, auto_speed_reg: Storage for auto settings

- prewash_en, extra_rinse_en: Internal option flags

- temp_saved_reg, speed_saved_reg, options_saved_reg: Internal configuration flags

- time_selected, time_remaining, count: Timing control values

- motor, water, drain, heat, door_locked: Internal control signals

- spin_phase_reg: Internal spin phase register

- temp_code: Selected temperature code

Functionality

- In AUTO_PRESET state: Sets parameters based on preset selection

  - Quick wash (000): 30°C, 1200 rpm, no prewash, no extra rinse

  - Shirts (001): 60°C, 800 rpm, no prewash, no extra rinse

  - Dark colors (010): 40°C, 1000 rpm, no prewash, with extra rinse

  - Dirty laundry (011): 40°C, 1000 rpm, with prewash, no extra rinse

  - Antiallergenic (100): 90°C, 1200 rpm, no prewash, with extra rinse

- In manual configuration states:

  - MANUAL_TEMP: Stores selected temperature

  - MANUAL_SPEED: Stores selected speed

  - MANUAL_OPTIONS: Stores selected options

- In READY state with start button pressed:

  - Calculates program time

  - Locks door

  - Sets initial washing sub-state

- In WASHING state:

  - Updates remaining time

  - Controls washing machine components based on current sub-state

  - Implements the complete washing cycle logic:

    - Fill water (activates water valve)

    - Prewash if enabled (motor at 60 rpm for 10 minutes)

    - Drain after prewash

    - Heat water (activates heater)

    - Main wash (motor at 60 rpm for 20 minutes)

    - Drain after main wash

    - Rinse (fill water, motor at 120 rpm for 10 minutes, drain)

    - Extra rinse if enabled

    - Spin (uses spin phases from decoder)

    - Complete (unlocks door after 1 minute)

6. Main Module (main.vhd)

Purpose

Top-level entity that connects all components together and interfaces with external inputs/outputs.

Signals

- External Inputs:

  - clk, reset: System clock and reset

  - start_btn: Start button

  - door_closed: Door closed sensor

  - mode_auto: Mode selection (auto/manual)

  - preset_sel: 3-bit preset selection

  - temp_sel, speed_sel: 2-bit temperature and speed selections

  - prewash_btn, extra_rinse_btn: Option buttons

  - config_saved: Configuration saved signal

- External Outputs:

  - ssd_display: 7-segment display output

  - state_leds: State LED outputs

Internal Signals

- State signals: current_state, next_state, washing_substate

- Configuration signals: manual_temp_reg, manual_speed_reg, auto_temp, auto_speed, etc.

- Selection signals: selected_temp, selected_speed

- Option signals: prewash_enabled, extra_rinse_enabled

- Configuration flags: temp_saved, speed_saved, options_saved

- Time signals: selected_time, remaining_time, counter

- Machine control signals: motor_on, water_valve, drain_valve, heater, door_lock

- Spin control signals: spin_phase, spin_outputs

Functionality

- Instantiates all four component modules

- Routes signals between components

- Implements signal assignments like the multiplexers for temperature and speed selection

Relationship Between Files

The components work together as follows:

- The package provides shared types and functions used by all other modules

- The main module connects all components together and handles external I/O

- The state transition module determines the next state based on inputs

  - Its output influences the signal handler and display controller

- The signal handler module controls the washing machine based on the current state

  - It outputs control signals and updates washing substates

  - It tracks timing and configuration parameters

- The display controller module shows relevant information on the display

  - It updates the display based on current state and parameters

- The spin decoder module provides specialized control during spin cycles

  - It decodes the spin phase into specific control signals

Together, these modules implement a complete washing machine controller that follows the specified requirements and organigram, providing both manual and automatic modes with appropriate control of all washing machine components.